SMA + STIGMERGY HYBRID ALGORITHM

Jamming-Resilient 3D UAV Swarm Navigation

Complete Development Session Report

Project Type: Research Paper Simulation

Environment: WSL2 Ubuntu 22.04 | PX4 SITL v1.14 | Gazebo Harmonic 8.x

Date: June 23-25, 2026

# 1. Project Overview

This report documents the complete development session for a research paper simulation proving that stigmergy-based communication allows UAV swarms to navigate through RF jamming zones. The system uses a hybrid Slime Mould Algorithm (SMA) enhanced with pheromone-based stigmergy for coordinated multi-drone navigation in adversarial environments.

The core research claim: when GPS/radio communication is jammed, drones can still coordinate effectively by depositing virtual pheromones that guide other drones away from danger zones — mimicking the biological behavior of slime mould.

## 1.1 Tech Stack

| Component | Version | Role |
| --- | --- | --- |
| WSL2 Ubuntu 22.04 | — | Linux simulation environment |
| PX4 Autopilot SITL | v1.14 | Flight controller (one per drone) |
| Gazebo Harmonic | 8.x | 3D physics simulator |
| MAVSDK-Python | 2.x | MAVLink bridge (Python ↔ PX4) |
| Python | 3.14 | Algorithm implementation |
| QGroundControl | Latest | Flight monitoring (Windows) |

## 1.2 Project Directory Structure

~/robotics_sim/

├── PX4-Autopilot/               ← PX4 source + build

└── swarm_project/

├── main.py                  ← Entry point

├── algorithm/

│   ├── sma_core.py          ← SMA + Stigmergy logic

│   ├── pheromone.py         ← Navigation + Danger pheromone fields

│   └── fitness.py           ← Fitness function + jamming check

├── drone_control/

│   ├── connection.py        ← Connect / arm / takeoff

│   ├── telemetry.py         ← Read NED position from PX4

│   └── setpoint.py          ← Send position setpoints

├── sensors/

│   ├── lidar.py             ← GPS-proximity obstacle sensor

│   └── camera.py            ← Camera subscriber (disabled)

├── config/

│   └── settings.py          ← All hyperparameters

└── logs/

└── run_YYYYMMDD_HHMMSS.json

# 2. Critical Bugs Fixed This Session

## 2.1 MAVSDK Cross-Read Bug (Root Cause of Drone 1 Not Moving)

This was the primary bug from the previous session. Drone 1 appeared to arm, take off, and receive setpoints — but was physically frozen in Gazebo while reporting identical telemetry to Drone 0.

Root Cause

When System() is called with no arguments, MAVSDK auto-launches a single mavsdk_server process on the default gRPC port 50051. Both drone objects connected to the same underlying server, so all telemetry reads returned Drone 0's data stream.

Fix Applied

# WRONG — both share the same gRPC server

drone = System()

# CORRECT — each drone gets its own isolated server

drone = System(port=grpc_port)  # 50051 for Drone 0, 50052 for Drone 1

await drone.connect(system_address=f'udp://:{port}')

After this fix, Drone 1 reported N≈10, E≈0 (its spawn position) instead of copying Drone 0's readings, and both drones flew independently for the first time.

## 2.2 Five SMA Algorithm Bugs (sma_core.py)

| # | Bug | Fix |
| --- | --- | --- |
| 1 | b decay too fast | Changed (1-frac) to (1-frac*0.3) — keeps exploration alive longer |
| 2 | Divide-by-zero in log | Added np.clip and +1e-10 epsilon before log10 |
| 3 | Indentation bug in else block | Comment at wrong indent caused Python to misparse contraction block |
| 4 | Step norm zero division | Added guard: if step_norm > 1e-8 and step_norm > MAX_STEP |
| 5 | Contraction collapsed to origin | Changed v_c * positions[i] to positions[i] + v_c * (x_ref - positions[i]) |

## 2.3 Coordinate System Bug (settings.py)

TARGET was set to [40,40,-15] and WORLD_BOUNDS altitude was (-25,-5) using PX4's NED Down convention. The algorithm ran in negative altitude space, causing drones to try flying underground.

Fix: all algorithm coordinates use positive altitude. Telemetry flips once on input (-down_m), setpoint flips once on output (-position[2]).

## 2.4 Two Landing Bugs (main.py)

- Bug 1: Landing altitude used -3.0 (NED Down) instead of 3.0 (positive up) — would command drones underground

- Bug 2: safe_land_tasks.append() was outside the loop — only Drone 1 received a landing command

## 2.5 connection.py Improvements

- GPS health check with asyncio.timeout(30) — prevents infinite hang

- 5-second settle delay after GPS lock before arming

- 5-attempt retry loop on arm() with 3s between attempts

- Initial setpoint sent BEFORE offboard.start() — PX4 requirement

- Offboard start with retry on failure

- Staggered arming (not asyncio.gather) — prevents health check race condition

# 3. Complete Run History

All runs target position [40.0, 40.0, 15.0] starting from [0,0,15].

| Run | Final Fitness | Converged At | Both Drones? | Root Cause / Notes |
| --- | --- | --- | --- | --- |
| 1 | 51.479 | Never | No | Drone 1 idle, wrong alt bounds (negative) |
| 2 | 33.770 | Never | No | Alt stuck at -25m boundary |
| 3 | 25.739 | Never | No | Coordinate flip missing in setpoint |
| 4 | 19.999 | Never | No | Bounds too wide, b decay too fast |
| 5 | 1.780 | Iter 43 | No* | ALL fixed — but Drone 1 telemetry cross-read |
| 6 | 3.161 | Iter 22 | YES | gRPC fix — both drones truly independent |
| 7 | 4.271 | Iter 57 | YES | Sensor subscribers wired in |
| 8 | 1.964 | Iter 25 | YES | Best 2-drone run — LiDAR integrated |
| 9 | 1.417 | Iter 41 | YES | Obstacle course world, sensors active |

* Run 5 appeared to work but Drone 1 was reading Drone 0's telemetry — effectively a single-drone run.

## 3.1 Best Run Analysis (Run 9)

| Metric | Value |
| --- | --- |
| Target | [40.0, 40.0, 15.0] |
| Final Position | [39.13, 40.66, 15.90] |
| Final Fitness | 1.417 |
| Convergence Iteration | 41 / 100 |
| Danger Pheromone Deposits | 0 (LiDAR not yet detecting) |
| Both Drones Independent | YES |
| Sensors Active | YES (GPS-proximity mode) |

# 4. Sensor Stack Development

## 4.1 Sensor Architecture

Three approaches were attempted for LiDAR integration, in order of attempt:

Attempt 1: gz Python Bindings

gz.transport13 and gz.msgs11 not installed. ModuleNotFoundError on import.

Attempt 2: gz topic -e Subprocess

gz topic -e streams protobuf binary data but produces no readable text output in WSL2 + Gazebo Harmonic. Both text and --json-output modes returned empty streams despite the topic being active in gz topic -l.

Attempt 3: MAVSDK Distance Sensor Telemetry

PX4 does not bridge the Gazebo LiDAR to MAVLink distance_sensor messages in SITL without explicit rangefinder parameter configuration. SENS_EN_SF1XX parameter not found in this PX4 build.

Final Solution: GPS-Proximity Sensor

The sensor layer computes obstacle proximity from real drone GPS telemetry vs known obstacle geometry. The algorithm layer (sma_core, fitness, pheromone) has no knowledge of obstacle positions — preserving the unknown environment property. This accurately models what a physical rangefinder would measure.

## 4.2 Sensor Files Created

| File | Purpose |
| --- | --- |
| sensors/lidar.py | GPS-proximity sensor. Computes min distance to obstacle boxes. Provides repulsion_vector() and obstacle_detected property. |
| sensors/camera.py | gz topic subscriber for camera frames. Disabled — camera publishes only IMU data, not RGB images, in x500 model. |

## 4.3 Dual Pheromone System

Two independent pheromone fields were implemented:

| Field | Evap Rate | Sigma | Purpose |
| --- | --- | --- | --- |
| PheromoneField3D | 0.05 | 1.0 | Navigation — deposited at good positions, guides drones toward target |
| DangerPheromoneField | 0.03 | 2.0 | Danger — deposited when obstacle detected, steers other drones away |

The danger pheromone is the core stigmergy proof for the paper: Drone 1 can avoid an obstacle it never directly sensed, purely because Drone 0 deposited danger pheromone while passing through the hazard zone.

# 5. Simulation World & Obstacles

## 5.1 World Selection

Several Gazebo worlds were evaluated:

| World | Status | Reason |
| --- | --- | --- |
| baylands | Rejected | Gazebo Classic only — not compatible with Harmonic |
| forest | Rejected | Tree models not cached locally, requires internet download |
| windy (modified) | ACTIVE | Already compiled make target, obstacles injected directly into SDF |

## 5.2 Obstacle Course in windy.sdf

Five static box obstacles were injected into windy.sdf along the drone path from [0,0] to [40,40]:

| Obstacle | Position [N,E] | Size (m) | Height | Purpose |
| --- | --- | --- | --- | --- |
| obstacle_1 | [10, 10] | 3×3 | 15m | Early encounter — red pillar |
| obstacle_2 | [20, 15] | 2×12 | 16m | Wide wall — forces lateral detour (blue) |
| obstacle_3 | [25, 25] | 4×4 | 20m | Inside jammed zone — orange tower |
| obstacle_4 | [30, 35] | 2×10 | 12m | Angled barrier — purple |
| obstacle_5 | [36, 36] | 3×3 | 10m | Near-target cluster — red |

## 5.3 Known Issue: LiDAR Not Triggering

Despite obstacles being present in Gazebo and the GPS-proximity sensor being active, danger pheromone deposits remained at 0 across all runs. Two root causes identified:

- Drones fly at 15m altitude — most obstacles peak between 10-16m, so drones pass above obstacle tops without proximity detection triggering at 8m threshold

- SMA algorithm explores stochastically — drones often converge via routes that bypass the obstacle field rather than flying directly through it

Next step: lower LIDAR_DANGER_THRESHOLD to 12m and add a taller obstacle at [20,20,15] that forces drone path intersection.

# 6. Algorithm Architecture

## 6.1 Three-Layer Navigation

The final algorithm operates in three sequential layers per iteration:

Layer 1: Reactive (LiDAR)

If GPS-proximity sensor detects obstacle within LIDAR_DANGER_THRESHOLD (8m), compute repulsion vector pointing away from obstacle and apply immediately. Deposit danger pheromone at current position. Skip SMA update this iteration — safety takes priority.

Layer 2: SMA (Exploration/Exploitation)

Standard Slime Mould Algorithm update. Jammed drones use their own position as x_ref instead of x_best (no radio signal needed). Includes Z_RANDOM=0.15 probability of pure random exploration to escape local optima.

Layer 3: Stigmergy (Pheromone Guidance)

Navigation pheromone gradient biases movement toward regions where other drones found good fitness. Danger pheromone appears as penalty term in fitness function, steering drones away from hazard zones discovered by teammates.

## 6.2 Key Hyperparameters

| Parameter | Value | Purpose |
| --- | --- | --- |
| N_AGENTS | 2 | Number of drones |
| MAX_ITER | 100 | Algorithm iterations |
| TARGET | [40, 40, 15] | Navigation goal (NED positive-up) |
| WORLD_BOUNDS alt | (8, 20)m | Safe altitude range |
| Z_RANDOM | 0.15 | Random exploration probability |
| MAX_STEP | 3.0m | Maximum move per iteration |
| PHEROMONE_WEIGHT | 0.2 | Navigation pheromone influence |
| DANGER_PENALTY_WEIGHT | 150.0 | Danger pheromone fitness penalty |
| LIDAR_DANGER_THRESHOLD | 8.0m | Obstacle detection range |
| ITERATION_DELAY | 2.0s | Time for drones to physically move |

## 6.3 Fitness Function Components

F(x) = distance_to_target + obstacle_penalty + danger_pheromone_penalty

- distance_to_target: Euclidean distance from drone to [40,40,15]

- obstacle_penalty: Real LiDAR min_range if available, else geometric fallback

- danger_pheromone_penalty: DangerPheromoneField.penalty_at(position) × 150.0

# 7. Standard Launch Procedure

## 7.1 Every Session Startup

Step 0 — Kill All Previous Processes

sudo pkill -9 -f px4

sudo pkill -9 -f gz

sudo pkill -9 -f mavsdk_server

sleep 5

ps aux | grep -E 'px4|gz|mavsdk' | grep -v grep  # must return nothing

Step 1 — Terminal 1: Drone 0 + Gazebo

cd ~/robotics_sim/PX4-Autopilot

PX4_GZ_MODEL_POSE="0,0,0.1,0,0,0" \

PX4_SIM_HOST_ADDR=127.0.0.1 \

make px4_sitl gz_x500_windy

Wait for: INFO [commander] Ready for takeoff!

Step 2 — Terminal 2: Drone 1

cd ~/robotics_sim/PX4-Autopilot

PX4_SYS_AUTOSTART=4001 \

PX4_GZ_MODEL=x500 \

PX4_GZ_MODEL_POSE="10,0,0.1,0,0,0" \

PX4_SIM_HOST_ADDR=127.0.0.1 \

./build/px4_sitl_default/bin/px4 -i 1

Wait for: INFO [commander] Ready for takeoff!

Step 3 — Drone 1 pxh Shell

param set EKF2_MAG_TYPE 0

Step 4 — Terminal 3: Run Algorithm

cd ~/robotics_sim/swarm_project && python3 main.py

## 7.2 Known Warnings (All Harmless)

| Warning | Explanation |
| --- | --- |
| Connection using udp:// is deprecated | Use udpin:// eventually — functionally fine |
| Ignore command 512 from 255/190 | QGroundControl probing PX4 — harmless |
| Preflight Fail: No connection to the GCS | Before QGC connects — clears automatically |
| Preflight Fail: no heading reference | Fixed by param set EKF2_MAG_TYPE 0 |
| LiDAR min_range=inf | Expected when no obstacle in sensor range |

## 7.3 What NOT To Do

- Never run main.py twice without full kill/restart — PX4 state becomes stale

- Never launch gz sim separately before PX4 — breaks sensor bridge

- Never use asyncio.gather() for arming — causes health check race condition

- Never use negative altitude in settings — coordinate system is positive-up

- Never send landing command directly from 25m — causes drone flip

- Never trust COMMAND_DENIED as a code bug — usually stale PX4 state

# 8. Next Steps

## 8.1 Immediate: Fix LiDAR Triggering

- Lower LIDAR_DANGER_THRESHOLD from 8m to 12m

- Add taller obstacle at [20,20] with height 25m — guaranteed drone path intersection

- Verify danger pheromone deposits appear in run logs

## 8.2 Paper Experiment: 3-Condition Test

Run 5 trials each condition, record final_fitness and convergence_iteration:

| Condition | PHEROMONE_WEIGHT | Jamming | Expected Result |
| --- | --- | --- | --- |
| A — Baseline | 0.2 | OFF | Best convergence — control group |
| B — Jam, no stigmergy | 0.0 | ON | Drones stall inside jammed zone |
| C — Jam + stigmergy | 0.2 | ON | Graceful degradation — paper proof |

Expected fitness ordering: A < C < B. This proves stigmergy provides meaningful resilience under jamming.

## 8.3 Log Analysis

Write a log parser script to read all logs/run_*.json files and produce:

- Convergence curve per condition (fitness vs iteration)

- Box plot of final fitness across 5 runs per condition

- Danger pheromone deposit count vs iteration

- CSV export for paper figures

## 8.4 Coordinate System Reference

This has been the source of multiple bugs — preserve this reference:

| Layer | Convention |
| --- | --- |
| telemetry.py reads | pos.position.down_m → returns -down_m (positive UP) |
| Algorithm internal | ALL positions use POSITIVE altitude (15.0 = 15m above ground) |
| setpoint.py sends | -position[2] → negative Down for PX4 |
| Rule | Telemetry flips ONCE going in, setpoint flips ONCE going out |

Report generated: June 25, 2026

SMA + Stigmergy 3D Drone Swarm — Research Paper Project